Cairo University

Faculty of Engineering

Electronics & Communication Department

ELC3050 Project

Design and Analysis of a 2-Element Probe-Fed Microstrip Patch Antenna Operating at 20 GHz

Under supervision of Dr Islam Eshra

| Name | ID |
| --- | --- |
| يوسف خالد عمر محمود | 9220984 |
| مصطفى أحمد حسني محمود درويش | 9220832 |
| مصطفى فتحي محمد حسن عسكر | 9220844 |
| يوسف اشرف محمد | 9220972 |
| عمر احمد رجب | 9220513 |
| عمر ايمن امين | 9220528 |

Table of Contents

Table of Figures	3

1. Introduction and Problem Description	5

Verification Against Another Source	5

Benchmark Description	5

2. Design Procedure	9

Bandwidth Enhancement Techniques:	28

3. Results and Discussion	35

3.1 Return Loss (S11)	35

3.2 Mutual Coupling (S21)	35

3.3 Smith Chart Analysis	35

3.4 Radiation Patterns	35

3.5 Gain and Efficiency	36

4. Final Design Layout	36

5. Conclusion	36

# Table of Figures

Figure 1: dipole antenna designed for verification	5

Figure 2: S11 for dipole antenna for verification	7

Figure 3: Gain 2D for dipole antenna for verification	7

Figure 5: Zin for dipole antenna for verification	8

Figure 4: Gain 3D for dipole antenna for verification	8

Figure 6: antenna with probe feeding model	12

Figure 7: 3D patch antenna design	12

Figure 8: bottom view of patch antenna	13

Figure 9: side view of patch antenna	13

Figure 10: S11 for single Patch antenna	13

Figure 11: Bandwidth where S11<-10 dB	14

Figure 12: smith chart for single Patch	14

Figure 13: VSWR = 1.421 at 20 GHz for single patch antenna	14

Figure 14: Xfeed for single patch antenna	15

Figure 15: Zin for single Patch antenna at 20GHz	15

Figure 16: Yfeed for single patch antenna	16

Figure 17: Electric Field Radiation for single patch antenna	16

Figure 18: magnetic field radiation for single patch antenna	18

Figure 19: Co-polarization and cross-polarization patterns in the E- and H-planes.	18

Figure 20: Gain 2D for single patch antenna	19

Figure 21: Gain 3D for single patch antenna	19

Figure 22: Results for one patch antenna	20

Figure 23: Two Patch antenna array	20

Figure 24: Two Patch side view	21

Figure 25: S11 for Two patches	21

Figure 26: Zin for two Patches at 20 GHz	21

Figure 27: VSWR for first Patch	22

Figure 28: VSWR for second Patch	22

Figure 29: S21 Vs Frequency	23

Figure 30: S21 sweep Vs frequency by changing dp	23

Figure 31: S21 Vs Distance between two patches	24

Figure 32: Electric field radiation of two patches	24

Figure 33: magnetic pattern for two patches	25

Figure 34: Co-polarization and cross-polarization patterns in the E- and H-planes.	25

Figure 35: Gain 2D for two patches	26

Figure 36: Gain 3D for two patches	26

Figure 37: Gain Vs Frequency	27

Figure 38: Radiation Efficiency Vs Frequency	27

Figure 39: T-Section transmission line	28

Figure 40: S11 after adding T-section TL showing enhancement in BW	29

Figure 41: Zin after adding T-section transmission line	29

Figure 42: VSWR after adding feeding network	30

Figure 43: magnetic Field Radiation of 2 antenna design probe feed	31

Figure 44: Electric Field Radiation of 2 antenna design probe feed	31

Figure 45: Co-polarization and cross-polarization patterns in the E- and H-planes for 2 antenna array probe feed	32

Figure 46: gain 2D for 2 antenna array probe feed	32

Figure 47: Gain 3D for 2 antenna array probe feed	33

Figure 48: table of final results	34

# 1. Introduction and Problem Description

This project involves the design of a 2-element probe-fed microstrip patch antenna operating at 20 GHz. The goal is to achieve an S11 less than -10 dB at the operating frequency while optimizing performance in terms of bandwidth, gain, and radiation efficiency. A comprehensive analysis of the antenna's mutual coupling and gain vs. element spacing is also included.

# Verification Against Another Source

We’re going to simulate a dipole with the design in figure 9.9 in refrence [1]

With values that’s applied in figure 1:

| Name | Unit | "Evaluated Value" | Description |
| --- | --- | --- | --- |
| dl | mm | 139.5mm | Antenna length |
| rdi | mm | 0.2625mm | Antenna radius |
| gap_L | mm | 1mm | Gap length |

Figure 1: dipole antenna designed for verification

## Benchmark Description

- A dipole antenna is a standard reference in antenna theory, with well-documented characteristics such as impedance, radiation pattern, and gain.

- It is widely used as a baseline for verifying simulation accuracy and comparing performance metrics.

Simulation Setup

- Design Parameters:

  - Length of the dipole: L=λ/2 λ is the wavelength at the operating frequency.

  - Material: Mention the conductor used (copper or PEC).

- Simulation Environment:

  - Define the simulation parameters, such as mesh size, boundary conditions, and excitation type (e.g., lumped port or wave port).

- Performance Metrics Evaluated:

  - Return Loss (S11).

  - Radiation patterns in E-plane and H-plane.

  - Gain and efficiency.

Equations Controlling Dipole Antenna

- Directivity:

D=1.64 (2.15 dBi for a half-wavelength dipole).

- Radiation Pattern:

  - E-plane pattern: E(θ)=E0sin(θ) for 0≤θ≤π.

  - H-plane pattern: E(ϕ)=constant for 0≤ϕ≤2π.

- Input Impedance:

Zin=Rr+jX, where X is the reactance.

- Gain:

G=D×η

where η is the efficiency of the antenna.

Results Comparison

We are going to use Table 9.1 in refrence [1] to verfy. According to it I’m expecting Rin=67

Figure 2: S11 for dipole antenna for verification

Figure 3: Zin for dipole antenna for verification

Figure 4: Gain 2D for dipole antenna for verification

Figure 5: Gain 3D for dipole antenna for verification

Conclusion

- The dipole antenna serves as a reliable reference for verifying the EM simulation tool.

- imulated results of the dipole antenna matches theoretical expectations (S11 plot, radiation pattern, and gain).

- The Rin is 68.63 which is approximately equal to the theotical value in Table 9.1.

So the EM tool is verified

# 2. Design Procedure

The design started with the selection of the substrate material R04003C with a dielectric constant of 3.55. Initial dimensions were calculated using standard formulas for microstrip patch antennas, considering a substrate thickness of 0.406 mm. An online calculator was used to determine the initial patch dimensions, which were fine-tuned through simulation sweeps for optimal S11 performance.

A single patch antenna was first designed and analyzed to establish baseline performance metrics. Subsequently, a 2-element array was constructed with varying patch separation distances (dp) to study mutual coupling. A matching network was designed for probe feeding to further optimize the design.

At first, we started with the following mathematical modelling for our design then we tuned and swept parameters to achieve required specs.

## Patch:

The resonant frequency of a rectangular microstrip patch antenna can be calculated using:

fr= c2Leffεeff

where:

- c: Speed of light in free space ( 3×108m/s )

- Leff: Effective length of the patch

- εeff: Effective dielectric constant of the substrate.

Effective Length:

- Leff=L+2∆L

where:

- ∆L=0.412.h (εeff+0.3)(w/h+0.264)(εeff-0.258)(w/h+0.8)

## Substrate:

Effective dielectric constant:

εeff=εr+12+εr-121+12hW-12

where:

- εr: Relative permittivity of the substrate

We used RO4003C dielectric with εr=3.55

- h: Height of the substrate

- W: Width of the patch.

Figure 6 RT-duroid 5870 - 5880 Data Sheet

2. Bandwidth Enhancement Analysis

Bandwidth (BW) is related to the quality factor (Q) by:

BW= frQ

Technique used to improve bandwidth:

- Impedance Matching: Adding a matching network to reduce reflections.

  - Use Zin and Z0 to compute matching network:

  - Γ= Zin-Z0Zin+Z0

3. Input Impedance

The input impedance at the feed point is given by:

Zin=Rin+jXin  ​

where Rin​ and Xin ​ are resistance and reactance components derived from field distributions.

4. Radiation Pattern

The far-field electric field components can be approximated as:

Eθ= jηI02πrsinθcos(kL2sinθ)

E​ϕ=0

where:

- k=2πλ ​: Wave number

- r: Distance to observation point

- η: Intrinsic impedance of the medium.

5. Gain and Efficiency

Gain (G) and radiation efficiency (ηr ​) are related:

G=ηr⋅D

where D is the directivity.

Efficiency:

ηr=PradPinput ​​

# Design of single patch

Figure 7: antenna with probe feeding model

Figure 8: 3D patch antenna design

Figure 9: bottom view of patch antenna

Figure 10: side view of patch antenna

And we designed and evaluated these values:

| Name | Unit | "Evaluated Value" | Description |
| --- | --- | --- | --- |
| Lp | Mm | 3.633mm | Patch length |
| Wp | Mm | 5.173mm | Patch width |
| hs | Mm | 0.406mm | Substrate height |
| Ws | Mm | 7.609mm | Ground plane width |
| Ls | Mm | 6.475mm | Ground plane length |
| xfeed | Mm | 1.2mm | Feed point x-offset |
| rcoax | Mm | 0.14mm | Coaxial feed radius |
| hcoax | Mm | 0.203mm | Coaxial feed height |
| rprope | Mm | 0.07mm | Probe radius |
| Yfeed | Mm | 0mm | Feed point y-offset |
| Hgnd | Mm | -0.032mm | Ground plane height |

### S11:

Figure 11: S11 for single Patch antenna

For designing single patch antenna, we tuned parameters to get reflection coefficient S11 achieving specs

From figure 11, we succeeded to tuned parameters and achieve minimum s11 at operating frequency 20 GHz =-15.35dB

we have bandwidth (range of frequency where S11<-10 dB) as shown  in figure 12.

Figure 12: smith chart for single Patch

Figure 13: Bandwidth where S11<-10 dB

As shown in figure 12, The BW = 450 MHz.

As shown in figure 14, Bandwidth achieved for VSWR=1.421

Figure 14: VSWR = 1.421 at 20 GHz for single patch antenna

### Zin:

Figure 15 Xfeed for single patch antenna

Figure 16 Zin for single Patch antenna at 20GHz

For proper feeding from the above figure 16, we found that Zin=41.98 is matching impedance required at 20GHz.

Figure 17: Yfeed for single patch antenna

From figure 15 and 17, We can see that (x,y) position can change the Zin value.

So we chose (1.2,0) that matched our specs.

Figure 18: Electric Field Radiation for single patch antenna

Radiation patterns

Figure 19: magnetic field radiation for single patch antenna

Figure 20: Co-polarization and cross-polarization patterns in the E- and H-planes.

Figure 21: Gain 2D for single patch antenna

Figure 22: Gain 3D for single patch antenna

Figure 24: Two Patch antenna array

But we are targeting to design 2 antenna arrays, so we made a replica from our patch and we swept and tuned our parameter to achieve the required specs

Figure 25: Two Patch side view

Also designing two patch antenna array is expected to provide us with higher gain compared to single patch however we noticed that there Is a trade off between mutual coupling S21 and achieving required gain :

At first, we tuned parameters to achieve S11 as required

Figure 26: S11 for Two patches

We have S11 = -10.46 dB< -10 dB at 20 GHz however we faced problems with Bandwidth as it is a very narrow band.

Figure 27: Zin for two Patches at 20 GHz

Figure 28: VSWR for first Patch

Figure 29: VSWR for second Patch

Figure 30: S21 Vs Frequency

as we discussed mutual coupling S21 originated when we add the second patch and we noticed that it varies with distance between two patches (dp) so we made sweep on it

Figure 31: S21 sweep Vs frequency by changing dp

Figure 32: S21 Vs Distance between two patches

we found that distance between two patches (dp=0.306 mm) to keep S11< -11 dB.

Figure 33: Electric field radiation of two patches

Figure 34: magnetic pattern for two patches

Figure 35: Co-polarization and cross-polarization patterns in the E- and H-planes.

Figure 36: Gain 2D for two patches

Figure 37: Gain 3D for two patches

as shown we noticed that gain of two patch antennas is larger than single patch antenna however it is inclined due interference (mutual coupling between two patches)

Figure 38: Gain Vs Frequency

Figure 39: Radiation Efficiency Vs Frequency

## Bandwidth Enhancement Techniques:

  - Impedance Matching: Adding a matching network to minimize reflections.

  - Stacked Patches: Introducing a second resonant patch above the main patch.

  - Capacitive Coupling: Modifying the feed structure to include a capacitive element.

  - Slotted Patch: Adding slots to the patch to create additional resonances.

To achieve bandwidth enhancement, we found that the previous techniques are used we started to design our antenna at given resonance frequency 20GHz and we got result for s11,VSWR,Radiation pattern, Gain and directivity, then we found that feeding network is not matched with designed antenna, so we decided to enhance bandwidth using Impedance Matching technique.

Adding Feeding Network for Two Patch antennas

Figure 40: T-Section transmission line

We got Zin=69.05 so for proper feeding and enhancing bandwidth we designed a transmission line T-section as shown below and we sweept on it’s dimensions till we achieved requirement.

Figure 41: S11 after adding T-section TL showing enhancement in BW

Figure 42: Zin after adding T-section transmission line

From the previous figure we enhanced bandwidth to be more wide from 19.23 GHz to 20.11 GHz

Where S11 is smaller than -10 dB.

Figure 43: VSWR after adding feeding network

Figure 44: magnetic Field Radiation of 2 antenna design probe feed

Figure 45: Electric Field Radiation of 2 antenna design probe feed

Figure 46: Co-polarization and cross-polarization patterns in the E- and H-planes for 2 antenna array probe feed

Figure 47: gain 2D for 2 antenna array probe feed

Figure 48: Gain 3D for 2 antenna array probe feed

| Name | Unit | "Evaluated Value" | Description |
| --- | --- | --- | --- |
| Lp | mm | 5.78mm | Patch length |
| Wp | mm | 7.54mm | Patch width |
| hs | mm | 0.406mm | Substrate height |
| Ws | mm | 11.736mm | Ground plane width |
| Ls | mm | 18.206mm | Ground plane length |
| xsub | mm | -9.103mm | Substrate center x-offset |
| ysub | mm | -5.868mm | Substrate center y-offset |
| xfeed | mm | 1.1mm | Feed point x-offset |
| dp | mm | 3.5mm | Patch offset parameter |
| rcoax | mm | 0.16mm | Coaxial feed radius |
| hcoax | mm | 0.203mm | Coaxial feed height |
| rprope | mm | 0.07mm | Probe radius |
| Lair | mm | 28.206mm | Airbox length |
| Wair | mm | 21.736mm | Airbox width |
| xair | mm | 14.103mm | Airbox x-center offset |
| yair | mm | 10.868mm | Airbox y-center offset |
| hair | mm | 5mm | Airbox height |
| yfeed | mm | 0mm | Feed point y-offset |
| xp | mm | 1.75mm | Patch x-origin |
| yp | mm | -3.77mm | Patch y-origin |
| hgnd | mm | -0.032mm | Ground plane height |
| Xcoax | - | 2 | Coaxial feed x-offset |
| WTL_In | mm | 0.673367mm | Input transmission line width |
| LTL_feed | mm | 2.179292mm | Feed transmission line length |
| WTL_feed | mm | 0.976256mm | Feed transmission line width |

Figure 49: table of final results

# 3. Results and Discussion

## 3.1 Return Loss (S11)

- The S11 parameter was evaluated for both the single patch and the 2-element array configurations. The single patch exhibited an S11 below -10 dB at the target frequency of 20 GHz, confirming adequate impedance matching. The 2-element array maintained a similar performance with an optimal patch separation distance of 0.36 mm.

- Importance of S11 < -10 dB: Achieving a return loss below -10 dB indicates that at least 90% of the input power is radiated, signifying efficient impedance matching and minimal reflections.

## 3.2 Mutual Coupling (S21)

- Mutual coupling between the patches was studied by sweeping the separation distance (dp). At dp = 0.36 mm, the coupling (S21) was minimized without significantly impacting the radiation characteristics.

- Element Spacing: Optimal spacing between array elements is vital to minimize mutual coupling, which can adversely affect radiation patterns and impedance matching. Studies suggest that a separation of approximately half a wavelength is effective in reducing coupling effects.

## 3.3 Smith Chart Analysis

- Impedance Matching: The Smith chart provides a visual representation of the antenna's impedance across frequencies. A locus close to the center of the chart at 20 GHz confirms effective matching, which is crucial for maximizing power transfer and minimizing signal reflections.

## 3.4 Radiation Patterns

- The co-polarization and cross-polarization patterns were analyzed in the E and H planes. The results demonstrated a directive radiation pattern with minimal cross-polarization, aligning with design expectations.

- E-plane and H-plane Patterns: Analyzing the radiation patterns in both planes reveals the antenna's directivity and beamwidth. A well-designed antenna should exhibit symmetrical patterns with minimal sidelobes, indicating efficient radiation and reduced interference.

- Cross-Polarization Levels: Low cross-polarization levels are essential for maintaining signal purity and reducing polarization mismatches, which is particularly important in communication systems to ensure signal integrity.

## 3.5 Gain and Efficiency

- Impact of Array Configuration: Transitioning from a single patch to a 2-element array can enhance gain due to constructive interference, but it's essential to manage mutual coupling to prevent efficiency degradation. Proper element spacing and feeding techniques are critical in this regard.

# 4. Final Design Layout

The final layout of the antenna, including the matching network and array configuration, is shown below:

# 5. Conclusion

The project successfully designed and analyzed a 2-element probe-fed microstrip patch antenna operating at 20 GHz. The design achieved the desired specifications with S11 below -10 dB, high gain, and radiation efficiency. Mutual coupling and gain vs. element spacing were thoroughly evaluated, and the results provide valuable insights for future antenna designs.

# 6. Refrences:

[1] C. A. Balanis, Antenna Theory: Analysis and Design, 4th ed. Hoboken, NJ: Wiley, 2016, Table 9.1.